BUILDING COST PLANNING AND COST INFORMATION MANAGEMENT IN SOUTH AFRICA

P.A. BOWEN(1) and P.J. EDWARDS(2)

(1)Department of Construction Economics and Management, University of Cape Town, Private Bag, Rondebosch 7700, South Africa and (2)Department of Building and Construction Economics, Royal Melbourne Institute of Technology, GPO Box 2476V, Melbourne, Australia

Abstract

The building cost planning and cost information management practices of professional quantity surveyors in South Africa are investigated through an opinion survey questionnaire. The survey deals with the nature and use of cost models and their associated cost data. The results indicate that quantity surveyors may be failing to adopt new techniques, and face the danger that both models and data may be used in inappropriate ways in cost planning future projects.

Keywords: Cost management, cost data, cost planning, quantity surveying.

Introduction

Building cost planning and cost information management services the information needs of building design at various stages during the pre-tender phase of the building procurement process. Cost planning is an expression of the economic implications of project design.

In South Africa, as in other countries which have adopted British practices, cost planing forms part of the consultancy services offered to building clients by professional quantity surveyors. The techniques of cost planning have been the frequent focus of  research over the past thirty years (e.g., RICS Wilderness Group, 1964; Raftery, 1984, 1987; Bowen, 1982, 1993; Bowen and Edwards, 1985; Newton, 1987; Akintoye et al., 1992).

Cost planning comprises an investigative process and an interpersonal communication process. The investigative process consists primarily of the application of a cost modelling technique; whereas the interpersonal communication process is mainly concerned with the communication of the derived information from the investigative process, i.e., the outcomes of cost planning (Bowen, 1993). It is important that the investigative process of cost planning not only uses appropriate techniques, but that it also employs appropriate data in servicing them.

The purpose of this research is to explore South African quantity surveyors' perceptions of their cost planning and cost information management practices, and to identify problems. The work builds upon that of Akintoye et al. (1992) and Tah et al. (1994).

The opinions of South African quantity surveyors were sought by means of a detailed questionnaire survey of 99 practitioners. The survey results are discussed in the context of the nature and use of cost models in South Africa and the cost data associated with them.

The nature and use of cost models in South Africa

In this section, the cost planning techniques used by quantity surveyors are presented under four main sub-headings: the cost models used; the stages at which particular models are used; and the perceived ability of the various models to model the construction process.

Usage of cost models

According to the survey responses, the most popular models, for cost planning purposes, are those which break down the building design into small packages, against which notional unit price rates can be applied in order to arrive at an aggregate estimate of building cost. Typically these are known as approximate quantities, and elemental cost planning methods. Most respondents frequently, or always, employ the approximate quantities (72%) or elemental (69%) cost modelling techniques.

These results correspond broadly with those of Akintoye et al. (1992), in their investigation into Nigerian building cost information management, although their findings show that fewer Nigerian quantity surveyors (21%) made frequent use of elemental cost models than their South African counterparts (69%). This is explained by the absence of a unique Nigerian element classification system.

Formal, mathematically-based building cost models are least favoured by South African quantity surveyors. Only 1% of respondents always or frequently use regression-based models, and only 3% always or frequently use expert systems in cost planning. The great majority of respondents never use either of these techniques.

While twelve practitioners (17%) stated that expert systems are used to some degree by their practices, there may be a lack of understanding on their part as to the exact nature of expert systems. Certain commercial building measurement (quantity take-off) packages (for example, "Take-Off") are marketed under the guise of "expert systems". It is conceivable that practitioners may be referring to these software systems. There is no evidence to suggest that a true expert system, dedicated to local quantity surveying practice, is currently in commercial use in South Africa.

Despite its presumed obsolescence (Sloan, 1996), 27% of South African quantity surveyor respondents frequently, or always, use the storey enclosure method of estimating the cost of building projects. This finding may be explained by the fact that this technique was the prescribed approach to cost planning for many public sector projects in South Africa during the 1970=s and early 1980=s. For this technique to have survived into the 1990's suggests that its adherents continue to place substantial trust in it. However, as no current cost data are published for this type of cost model, its adherents must rely completely on information generated "in-house". The issue of data source and availability is discussed later, in the context of the more popular cost models and their usage.

A surprising result from the survey was the finding that some 38% of respondents seldom, or never, use the spatial (cost/m2) cost model, and that only 19% of respondents use this technique more than occasionally. Despite its limitations, the cost per m2 of a building surely retains a simple attractiveness to clients and other members of the design team? In any event, it is likely that communication of the outcomes of  elemental cost planning will incorporate a statement of the cost/m2 in the report.

The usage pattern displayed by South African quantity surveyors suggests that they are reluctant to engage to any great extent in cost model development. No evidence was found in the survey to indicate development other than the straightforward transposition of manually-based techniques into computerised applications. For example, no respondent reported any attempt to integrate cost modelling techniques with computer-aided design processes.

Rationale for choice of model

The survey also explored explanations for the popularity of cost planning techniques. Six factors, that could possibly influence the choice of cost modelling technique employed by quantity surveyors, were suggested in the questionnaire. Respondents’ opinions regarding the extent of their influence are summarised in Table 1.

Although the option "other" was included as a factor variable, this is disregarded for the purpose of analysis as only one other possible factor (without explanation) was offered by respondents.

Availability of information; and the time available for the cost planning task, are the most frequently cited factors affecting quantity surveyors’ choice of cost modelling technique.

Table 1. Factors influencing quantity surveyors' choice of cost model (rank shown in parentheses)

| Factor | Response frequency for various levels of influence |  |  |
| --- | --- | --- | --- |
|  | Always / Frequently | Occasionally | Seldom / Never |
|  | (%) | (%) | (%) |
| Cost of project (4) | 55 | 17 | 28 |
| Size of project (3) | 64 | 15 | 22 |
| Client type (5) | 46 | 31 | 23 |
| Client sophistication (6) | 43 | 29 | 29 |
| Time available (2) | 74 | 15 | 11 |
| Information available (1) | 78 | 14 | 8 |

However, this rationale must be seen against the respondents’ overwhelming preference for approximate quantities and elemental cost planning techniques. Neither of these can be regarded as undemanding in terms of information, nor as particularly rapid in application compared to, say, functional unit or spatial cost models.

The inference to be drawn is that South African quantity surveyors may be rejecting faster, less Adata hungry@ cost modelling techniques for other reasons, such as perceived lack of reliability or theoretically complex mathematical formulation requirements (eg. regression models and expert systems). Other research evidence (Koh, 1996) indicates that practitioners prefer to Aforce fit@ one of the popular techniques to suit the available information and time constraints, rather than adopt a different technique.

Cost model usage at the various stages of the design process

It is not only necessary to establish the cost planning methods favoured by quantity surveyors, but also when these models are used during the design process. This model/design stage relationship is important for an understanding of the design/data/model interface (Raftery, 1987).

It has been established that South African quantity surveyors have a clear preference for using approximate quantities and/or elemental methods of cost planning. The survey responses have been used to explore usage of these cost models at various stages of the design process. The findings are shown in Table 2.

Table 2. Quantity surveyors' usage of approximate quantities and elemental cost

models during the design process.

| Design stage | Response frequency |  |
| --- | --- | --- |
|  | Approximate quantities | Elemental |
|  | (%) | (%) |
| Inception | 23 | 31 |
| Appraisal | 68 | 71 |
| Design concept | 85 | 85 |
| Design development | 68 | 65 |
| Documentation | 5 | 8 |

The similarity of the frequency responses for each of these models at each stage of the design process (with the exception of the inception stage) suggests that they may be being used in some form of combination. Again, the evidence of Koh (1996) supports this proposition. He explored quantity surveyors' compliance with Australian published cost control procedures. His survey of eleven Melbourne quantity surveying practices found that all respondents never or rarely used the published NPWC (1980) elemental system to the level of detailed sub-element (component) costs. These are replaced as soon as possible by the approximate quantities method. This occurs because of a lack of historical cost data for the more detailed levels of elemental cost planning, which in turn suggests that the effort required to provide these data through historical cost analysis of projects is not thought to be cost effective; hence the partial application of the elemental system to new projects. This supports Mooney's (1983) and Beeston's (1987) proposals for a more simplified elemental approach to cost planning, through reducing the number of individual elements (by appropriate element groupings); and by minimising the inclusion of detailed, but cost insignificant, items. Such simplification, however, relies on the willingness of quantity surveyors to delay the use of more detailed cost planning techniques until later in the design process, when better information becomes available. Apart from the inception stage, where spatial (costs per m2) cost models enjoy their highest usage, the survey research shows that quantity surveyors are not yet willing to do this, but prefer to adopt detailed techniques as quickly as possible.

Survey respondents were asked why particular cost modelling methods are preferred at various stages of the design process. The main reasons (with the preferred methods shown in parentheses at each stage of design) may be summarised as follows:

| : inception (superficial method) | - appropriate to the requirement of an approximate cost, commensurate with the amount of information and the time available for the forecast, easy to use |
| --- | --- |
| : appraisal (elemental and approximate quantities) | - accuracy of method, reliability, and time and level of information available |
| : design concept (elemental and approximate quantities) | - accuracy of method and amount of information available |
| : design development (approximate quantities and elemental) | - reliability of method and amount of information available |
| : documentation (bills of quantities) | - accuracy of method. |

These findings indicate that, once the inception stage of the design process has passed, the perceived accuracy and reliability of the cost model are the dominant reasons for South African quantity surveyors’ choice of cost model. This supports the earlier inference drawn from Table 1, and supports the notion that quantity surveyors may prefer to Aforce-fit@ a model to suit the time and level of information available, rather than change to a different model.

To summarise this section on the usage of cost models, it appears that, in keeping with the conclusions of Morrison (1983) and Stevens (1983) in the United Kingdom, South African quantity surveyors have clear preferences for certain modelling techniques at the various stages of the design process. They seek to implement as detailed a modelling technique as possible, as early as possible, notwithstanding the likely absence of suitably detailed design information. This is likely to require many assumptions regarding the anticipated specification of the building and the nature and extent of the design.

Cost model representation of the construction process

A cost model will best represent the construction process if it incorporates some measure of the construction resources needed to build the project, and several researchers (Beeston, 1987; Marston and Skitmore, 1990; Bowen, 1993) have advocated the adoption of resource-based approaches to cost modelling for building projects. Such approaches attempt to represent the construction process and reflect the anticipated use of construction resources. They are typified by techniques such as critical path planning and activity/operational bills of quantities.

Questioned about their use of resource-based cost models, eighty-three per cent of respondents indicated that they seldom, or never, utilise critical path methods or activity bills of quantities in their cost planning activities. At best, these methods are used only occasionally. This points to a gap between theory and practice which needs to be addressed.

Given their disinclination towards using resource-based cost models, it is necessary to examine quantity surveyors’ opinions about the extent to which construction processes are represented in the models they do favour.

The extent to which quantity surveyors consider that the various cost models are able to represent (model) the construction process is, to some extent, a measure of their faith in the prima facie validity of the model and its usefulness in the cost planning process. Table 3 represents quantity surveyors' opinions about the ability of the various cost models to represent the construction process.

The spread of responses for many of the modelling techniques (e.g., superficial, storey enclosure, comparative, regression, interpolation and expert systems), suggests that quantity surveyors differ widely in their opinions of the comparative representativeness of these techniques. Lack of direct knowledge may be the key to this, at least in part. With regression cost modelling, for example, it is possible to incorporate construction process-related variables in the derived model equations; but few quantity surveyors may be fully aware of this. This, in turn, suggests that researchers are not effectively reaching quantity surveying practitioners with information about developments in process-based cost models.

Table 3. Quantity surveyors' opinions regarding the relationship between cost model type and model ability to represent the construction process (rank shown in parentheses)

| Model type | Degree of representation |  |  |
| --- | --- | --- | --- |
|  | Very Good / Good | Acceptable | Poor / Very Poor |
|  | (%) | (%) | (%) |
| Functional unit (11) | 4 | 23 | 73 |
| Superficial (9) | 7 | 44 | 49 |
| Cubic (12) | 0 | 19 | 81 |
| Storey enclosure (7) | 18 | 38 | 45 |
| Approximate quantities (2) | 95 | 5 | 0 |
| Elemental (3) | 89 | 11 | 0 |
| Bills of quantities (1) | 100 | 0 | 0 |
| Lump sum (10) | 6 | 16 | 78 |
| Comparative forecasts (6) | 20 | 58 | 22 |
| Regression (5) | 27 | 42 | 32 |
| Interpolation (8) | 17 | 42 | 41 |
| Expert systems (4) | 42 | 42 | 16 |

There is a certain amount of unfairness in the survey question about the representative capability of the cost models, as none of the models listed in Table 3, with the exception of regression models and expert systems, are capable of representing the construction process explicitly. Bowen (1993) found that the construction resource cost implications of design are most often dealt with, from a cost planning point of view, by the inclusion of estimates of contract preliminaries. This would also explain respondents’ overwhelming preference for approximate quantities, elemental, and bills of quantities cost models as being most representative of the construction process, since these models usually incorporate separate estimates of contract preliminaries. However, at the deisgn cost planning stage, these estimates are usually based upon a historically- or intuitively-determined percentage of the net building cost (Bowen, 1993). The opportunity for close consideration of the resource requirements for particular projects is thus foregone. The time constraints upon cost planning, earlier indicated as an important factor by South African quantity surveyors, appears to preclude more detailed attention to the construction resource implications of buildings during the design stage.

Data used for cost models

The use of cost data is an essential element of the cost planning and control processes for building projects. This section documents South African quantity surveyors’ opinions about the nature and extent of the use of building cost data. It deals with the form and nature of the data recorded by quantity surveying practices; their data preferences; the sources of data; the forms of data used in association with the various cost modelling techniques; and the stages at which various data are used.

The survey results show that the two most popular forms of cost data are derived Ain house@, ie. item rates obtained from bills of quantities held by the practice, and elemental rates derived from analyses of previous commissions undertaken by the practice. The calculation of new rates from "first principles" is generally a less popular form of data than rates obtained from published price/cost books and lists.

The survey questionnaire went on to explore how frequently respondents make use of various types of cost data.

Table 4 indicates the frequency of South African quantity surveyors' recourse to various cost data sources. These results support the previous finding, and that of Akintoye et al. (1992) that quantity surveyors prefer cost data derived "in-house".

Table 4. Sources of data used in cost planning

| Form of data | Response frequency for various levels of data use |  |  |
| --- | --- | --- | --- |
|  | Always / Frequently | Occasionally | Seldom / Never |
|  | (%) | (%) | (%) |
| In-house data | 98 | 2 | 0 |
| Published price/cost books | 27 | 28 | 45 |
| Specialist quotations | 82 | 12 | 6 |

The fact that nearly half of the respondents seldom, or never, make recourse to published building price books is disturbing. It hints at an inward-looking cost data attitude which might be obsessive.

The data form/model type association

Asking respondents to associate the different forms of cost data with the various cost modelling techniques, produced the results shown in Table 5.

Because use of the various forms of data is not exclusive to any particular model, the actual number of respondents claiming to use certain data for particular models is given.

As with their Nigerian counterparts (Akintoye et al., 1992), South African quantity surveyors clearly favour the use of "in-house" data, augmented where necessary by specialist quotations, for all the models used in cost planning.

These findings, along with those relating to the popularity of the approximate quantities and elemental cost planning models, point towards the potential for a vicious cost modelling Aspiral@ for professional quantity surveyors in South Africa. Practitioners prefer to use models which are Adata hungry@ (ie. approximate quantities and elemental models), and are reluctant to use data other than derived Ain house@. Eventually this must lead to a lack of heterogeneity in the cost data used, and thus potentially to the use of data which are inappropriate for the cost planning of particular projects. Furthermore, it could also lead to the use of data which have passed their "use by" date, ie. quanity surveyors will prefer to hoard and use obsolescent cost data (eg. data representing outdated technologies) and update it by means of cost indices rather than seeking fresh data. A proclivity for Ain house@ cost data also tends to anchor South African quantity surveyors to the traditional form of building procurement, as this has provided their main source of cost data for many years.

Table 5. Forms of data associated with cost models (rank for in-house data shown in parentheses)

| Model type | Number of respondents for various forms of data |  |  |
| --- | --- | --- | --- |
|  | In-house | Price/cost books | Specialist quotations |
|  | (No.) | (No.) | (No.) |
| Functional unit (7) | 26 | 4 | 6 |
| Superficial (4) | 73 | 4 | 8 |
| Cubic (10) | 7 | 0 | 1 |
| Storey enclosure (8) | 25 | 0 | 6 |
| Approximate quantities (1) | 85 | 24 | 45 |
| Elemental (3) | 83 | 20 | 43 |
| Bills of quantities (1) | 85 | 23 | 54 |
| Lump sum (6) | 30 | 7 | 22 |
| Comparative (5) | 48 | 5 | 9 |
| Regression (11) | 6 | 0 | 1 |
| Interpolation (9) | 11 | 1 | 2 |
| Expert systems (12) | 4 | 0 | 4 |

The cost data form/design stage association

Survey participants were invited to indicate what types of data were most used at each of the various stages of the design process. The results are reflected in Table 6. The actual numbers of respondents are given, as explained above.

Table 6. Forms of data associated with various design stages

| Form of data | Number of respondents for various stages of design |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Inception | Appraisal | Design concept | Design development | Documentation |
|  | (No.) | (No.) | (No.) | (No.) | (No.) |
| In-house | 88 | 80 | 82 | 78 | 73 |
| Price/cost books | 10 | 19 | 22 | 20 | 17 |
| Specialist quotations | 18 | 40 | 57 | 64 | 61 |

The survey results show that the use of "in-house" data is consistent throughout the design process, diminishing marginally towards the documentation stage. To some extent, the danger of the cost modelling spiral, referred to above, is alleviated as the use of specialist quotations increases as the design phase progresses, peaking at the design development and documentation stages. The use of all three sources of data diminishes marginally as the documentation stage is reached, probably due to a reduction in the demand for cost planning advice by clients at this stage. It was not possible to establish, from the survey data, whether or not quantity surveyors prefer "in-house" data from projects with which they themselves have been personally involved, i.e., a partner or associate in a firm using only historical data from a project which she or he has worked on, but the personal experience of the authors suggests that this is not unusual.

In summary, the sources and preferences of building cost data, for South African quantity surveyors, are similar to those of practitioners elsewhere. "In-house" data, generally based upon analyses of historical projects, is preferred above all other forms. However, South African quantity surveyors are unaware of, or choose to ignore, the issues of data reliability raised by the smallness of their data sets. It was not possible, in this research, to explore the issue of inappropriate extrapolation of "in-house" cost data to new projects.

Conclusions

The cost information management practices of ninety-nine South African quantity surveyors have been investigated.

With respect to the nature and use of cost models during the cost planning process for building projects, the most popular models, in terms of usage, are approximate quantities measurement and elemental cost planning systems. These may be employed throughout most of the design appraisal, design concept and design development stages of the building procurement process. The superficial method and quantity surveyor-priced bills of quantities, are employed at more specific times; the former being most popular at project inception stage; and the latter being used almost exclusively at the conclusion of the project documentation stage.

South African quantity surveyors show little inclination to adopt models which can more closely represent the construction process itself, i.e., resource-based cost models. They seldom use technically advanced knowledge-based systems and regression cost models.

Despite the statistical limitations of such a practice, quantity surveyors in South Africa prefer to service their cost models with "in-house" data generated from previous projects undertaken by the firm. Intuitive, or "gut-feel" rates may be used early on in the design phase. Although most quantity surveying practices hold published construction price/cost books, little use is made of this form of cost data. In such a cost data environment, there is a real danger that South African quantity surveyors, as alternative procurement systems become more widely adopted, will increasingly be tempted to use Ain-house@ data in ways which are inappropriate. This is a serious issue which quantity surveyors have done little to address to date.

References

Akintoye, A.S., Ajewole, O. and Olomolaiye, P. (1992) Construction cost information management in Nigeria. Construction Management and Economics, Vol.10, No.2, pp.107-116.

Beeston, D.T. (1987) A future for cost modelling, in Building Cost Modelling and Computers (Ed. P.S. Brandon), E. & F.N. Spon Ltd., London, pp.17-24.

Bowen, P.A. (1982) An alternative estimating approach? Chartered Quantity Surveyor, February, pp.191-194.

Bowen, P.A. (1993) A Communication-based Approach to Price Modelling and Price Forecasting in the Design Phase of the Traditional Procurement Process in South Africa. Unpublished Ph.D. Thesis, University of Port Elizabeth.

Bowen, P.A. and Edwards, P.J. (1985) Cost modelling and price forecasting: theory and practice in perspective. Construction Management and Economics, Vol.3, pp.199-215.

Koh, K.G. (1996) The Use of, and Compliance with, NPWC Cost Control Procedures in the Building Industry. Unpublished Report, Department of Building and Construction Economics, Royal Melbourne Institute of Technology, Australia.

Marston, V.K. and Skitmore, R.M. (1990) Automatic resource based cost-time forecasts. Transactions, American Association of Cost Engineers, pp.M.6.1-M.6.6.

Mooney, J. (1983) Cost Effective Building Design. NSW University Press, Kensington, New South Wales.

Morrison, N.A.D. (1983) The Cost Planning and Estimating Techniques employed by the Quantity Surveying Profession. Unpublished Ph.D. Thesis, Department of Construction Management, University of Reading, Reading.

Newton, S. (1987) Computers and cost modelling: what is the problem?, in Building Cost Modelling and Computers (Ed. P.S. Brandon), E. & F.N. Spon, London, pp.41-48.

NPWC (1980) Cost Control Manual. Second Edition, National Public Works Council, Canberra.

Raftery, J.J. (1984) Models in building economics: a conceptual framework for the assessment of performance, in Proceedings of the C.I.B W-55 Third International Symposium on Building Economics, Ottawa, Vol.3, pp.103-111.

Raftery J.J. (1987) The state of cost/price modelling in the U.K. Construction Industry: a multicriteria approach, in Building Cost Modelling and Computers (Ed. P.S. Brandon), E. & F. N. Spon Ltd., London, pp.49-71.

Royal Institution of Chartered Surveyors (RICS) (1964) An Investigation into Building Cost Relationships. The Wilderness Cost of Buildings Study Group, Royal Institution of Chartered Surveyors, London.

Sloan, B. (1996) Review of `Building Economics' (4th Ed.) by I.H. Seeley. Journal of Financial Management of Property and Construction, Vol.1, No.1, pp.103-104.

Stevens, S. D. (1983) The Use of Price Data from Analysis of Bills of Quantities in Construction Price Estimating. Unpublished Ph.D. Thesis, Department of Construction Management, University of Reading, Reading.

Tah, J.H.M., Thorpe, T. and McCaffer, R. (1994) A survey of indirect cost estimating in practice. Construction Management and Economics, Vol.12, No.1, pp.31-36.